Experiment No.: 01

Title

Communication Between Two Networks Using Simple Router Configuration in Cisco Packet Tracer

Aim

To configure a Cisco router to establish communication between two different Local Area Networks (LANs) using static IP addressing in Cisco Packet Tracer.

Objectives

After completing this experiment, students will be able to:

- Understand the basic concept of routing.

- Configure IP addresses on PCs.

- Configure router interfaces using Cisco IOS commands.

- Verify communication between two different networks.

- Troubleshoot basic network connectivity problems.

Apparatus / Software Required

| Hardware/Software | Quantity |
| --- | --- |
| Cisco Packet Tracer | 1 |
| Cisco Router (1941/2911) | 1 |
| Cisco Switch (2960) | 2 |
| Personal Computer (PC) | 2 |
| Copper Straight-Through Cable | As Required |

Theory

A router is a Layer-3 networking device that connects multiple networks and forwards packets based on IP addresses. Every router interface belongs to a separate network and acts as the default gateway for devices connected to that network.

In this experiment, two LANs are connected through a router.

- LAN 1: 192.168.1.0/24

- LAN 2: 192.168.2.0/24

The router forwards packets between both networks, allowing communication.

Network Topology

IP Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC0 | NIC | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| Router | G0/0 | 192.168.1.1 | 255.255.255.0 | — |
| Router | G0/1 | 192.168.2.1 | 255.255.255.0 | — |
| PC1 | NIC | 192.168.2.10 | 255.255.255.0 | 192.168.2.1 |

Procedure

Step 1: Create the Network Topology

- Open Cisco Packet Tracer.

- Add one Cisco Router (1941).

- Add two Cisco 2960 switches.

- Add two PCs.

- Connect devices using Copper Straight-Through cables.

Connections

| From | To |
| --- | --- |
| PC0 | Switch0 |
| Switch0 | Router G0/0 |
| Router G0/1 | Switch1 |
| Switch1 | PC1 |

Step 2: Configure PC0

Desktop → IP Configuration

| Parameter | Value |
| --- | --- |
| IP Address | 192.168.1.10 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 192.168.1.1 |

Step 3: Configure PC1

Desktop → IP Configuration

| Parameter | Value |
| --- | --- |
| IP Address | 192.168.2.10 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 192.168.2.1 |

Step 4: Configure Router

Enter the following commands in the router CLI.

Configure Interface G0/0

Router> enable

Router# configure terminal

Router(config)# interface gigabitEthernet0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Configure Interface G0/1

Router(config)# interface gigabitEthernet0/1

Router(config-if)# ip address 192.168.2.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# end

Save Configuration

Router# copy running-config startup-config

Press Enter when prompted.

Verification Commands

Check Interface Status

show ip interface brief

Expected Output

| Interface | IP Address | Status |
| --- | --- | --- |
| G0/0 | 192.168.1.1 | Up |
| G0/1 | 192.168.2.1 | Up |

Check Routing Table

show ip route

Expected Output

C 192.168.1.0/24 is directly connected

C 192.168.2.0/24 is directly connected

Connectivity Test

From PC0

ping 192.168.2.10

From PC1

ping 192.168.1.10

Expected Result

Reply from destination.

Packets: Sent = 4, Received = 4, Lost = 0

Troubleshooting

| Problem | Cause | Solution |
| --- | --- | --- |
| Destination Host Unreachable | Wrong IP Address | Verify IP configuration |
| Request Timed Out | Router Interface Down | Use no shutdown |
| Cannot Ping | Incorrect Gateway | Verify Default Gateway |
| Interface Down | Cable Problem | Check Cable Connections |
| Routing Failure | Incorrect IP Address | Verify Router Configuration |

Result

Communication between two different networks was successfully established using a Cisco router. The PCs were able to communicate with each other through proper IP addressing and router configuration.

Viva Questions

- What is the purpose of a router?

- What is routing?

- Why is a default gateway required?

- What does the no shutdown command do?

- What is the function of the show ip interface brief command?

- What is the difference between a switch and a router?

- What is a subnet mask?

- What command saves the router configuration?

- Which OSI layer does a router operate on?

- What happens if the default gateway is configured incorrectly?